INSTRUCTIONS FOR VISOR REMOTE 0220 MODELS

Read the following instructions thoroughly before using the remote.

This transmitter must be adjusted so that the coding switches match the coding on your present transmitters or receiver

FCC Regulations: This device complies with FCC Rules Part 15.

Operation is subject to the following two conditions: 1) This device may not cause harmful interference. 2) This device must accept an interference that may cause undesired operation. Transmitter distance may vary due to circumstances beyond our control.

Remove the Transmitter Cover for battery replacement and set the Coding Switches

Figure A

Programming Remote Controller to the Opener

- Remove the transmitter cover

Remove the access cover from the front of the transmitter by sliding the cover away from the top of the transmitter (Figure A). This will expose the battery and the dip switches. The code can now be changed using a small screwdriver.

- Setting the codes

You may set your transmitter to any code you desire, but be sure that the code you set matches both on your transmitter and receiver.

There are (9) dip switches, each of which can be placed in three different positions (+, 0, -). DO NOT set all switches in the same positions such as all +, all -, or all zero (Figure A). WARNING: no other adjustments should be made inside the transmitter.

Warning

To avoid possible serious injury or death from garage door or moving gate:

-Keep all remotes out of children’s reach at all times. Never allow children near batteries!

-Only activate door or gate when it is in clear view and is in proper working condition.

-Always completely monitor garage door or gate opener until completely opened or closed.

-Never allow anyone to cross the opening or closing of moving garage door or gate.

-Changes or modifications to this unit not expressly approved by the party responsible for compliance could void the user’s authority to operate this equipment.

INSTRUCTIONS FOR VISOR REMOTE 0220 MODELS

Read the following instructions thoroughly before using the remote.

This transmitter must be adjusted so that the coding switches match the coding on your present transmitters or receiver

FCC Regulations: This device complies with FCC Rules Part 15.

Operation is subject to the following two conditions: 1) This device may not cause harmful interference. 2) This device must accept an interference that may cause undesired operation. Transmitter distance may vary due to circumstances beyond our control.

Remove the Transmitter Cover for battery replacement and set the Coding Switches

Figure A

Programming Remote Controller to the Opener

- Remove the transmitter cover

Remove the access cover from the front of the transmitter by sliding the cover away from the top of the transmitter (Figure A). This will expose the battery and the dip switches. The code can now be changed using a small screwdriver.

- Setting the codes

You may set your transmitter to any code you desire, but be sure that the code you set matches both on your transmitter and receiver.

There are (9) dip switches, each of which can be placed in three different positions (+, 0, -). DO NOT set all switches in the same positions such as all +, all -, or all zero (Figure A). WARNING: no other adjustments should be made inside the transmitter.

Warning

To avoid possible serious injury or death from garage door or moving gate:

-Keep all remotes out of children’s reach at all times. Never allow children near batteries!

-Only activate door or gate when it is in clear view and is in proper working condition.

-Always completely monitor garage door or gate opener until completely opened or closed.

-Never allow anyone to cross the opening or closing of moving garage door or gate.

-Changes or modifications to this unit not expressly approved by the party responsible for compliance could void the user’s authority to operate this equipment.

INSTRUCTIONS FOR VISOR REMOTE 0220 MODELS

Read the following instructions thoroughly before using the remote.

This transmitter must be adjusted so that the coding switches match the coding on your present transmitters or receiver

FCC Regulations: This device complies with FCC Rules Part 15.

Operation is subject to the following two conditions: 1) This device may not cause harmful interference. 2) This device must accept an interference that may cause undesired operation. Transmitter distance may vary due to circumstances beyond our control.

Remove the Transmitter Cover for battery replacement and set the Coding Switches

Figure A

Programming Remote Controller to the Opener

- Remove the transmitter cover

Remove the access cover from the front of the transmitter by sliding the cover away from the top of the transmitter (Figure A). This will expose the battery and the dip switches. The code can now be changed using a small screwdriver.

- Setting the codes

You may set your transmitter to any code you desire, but be sure that the code you set matches both on your transmitter and receiver.

There are (9) dip switches, each of which can be placed in three different positions (+, 0, -). DO NOT set all switches in the same positions such as all +, all -, or all zero (Figure A). WARNING: no other adjustments should be made inside the transmitter.

Warning

To avoid possible serious injury or death from garage door or moving gate:

-Keep all remotes out of children’s reach at all times. Never allow children near batteries!

-Only activate door or gate when it is in clear view and is in proper working condition.

-Always completely monitor garage door or gate opener until completely opened or closed.

-Never allow anyone to cross the opening or closing of moving garage door or gate.

-Changes or modifications to this unit not expressly approved by the party responsible for compliance could void the user’s authority to operate this equipment.

INSTRUCTIONS FOR VISOR REMOTE 0220 MODELS

Read the following instructions thoroughly before using the remote.

This transmitter must be adjusted so that the coding switches match the coding on your present transmitters or receiver

FCC Regulations: This device complies with FCC Rules Part 15.

Operation is subject to the following two conditions: 1) This device may not cause harmful interference. 2) This device must accept an interference that may cause undesired operation. Transmitter distance may vary due to circumstances beyond our control.

Remove the Transmitter Cover for battery replacement and set the Coding Switches

Figure A

Programming Remote Controller to the Opener

- Remove the transmitter cover

Remove the access cover from the front of the transmitter by sliding the cover away from the top of the transmitter (Figure A). This will expose the battery and the dip switches. The code can now be changed using a small screwdriver.

- Setting the codes

You may set your transmitter to any code you desire, but be sure that the code you set matches both on your transmitter and receiver.

There are (9) dip switches, each of which can be placed in three different positions (+, 0, -). DO NOT set all switches in the same positions such as all +, all -, or all zero (Figure A). WARNING: no other adjustments should be made inside the transmitter.

Warning

To avoid possible serious injury or death from garage door or moving gate:

-Keep all remotes out of children’s reach at all times. Never allow children near batteries!

-Only activate door or gate when it is in clear view and is in proper working condition.

-Always completely monitor garage door or gate opener until completely opened or closed.

-Never allow anyone to cross the opening or closing of moving garage door or gate.

-Changes or modifications to this unit not expressly approved by the party responsible for compliance could void the user’s authority to operate this equipment.