Programming instructions for OHD 451, 456, 551, 556, 656 

Uses Code Switch Remote
1. On the back of the motor press and release the far-right button labeled SC.

2. Light on the motor head will flash once.

3. Press the remotes button and light will flash twice.

4. Press the remote button and the light will flash 3 times.

5. Press the remote button and the light will flash 4 times.

6. Press the remote button and the light will flash 5 times.

7. Light will go out for about 15 seconds then come back on then off.

8. Programming completed and you can test the remote.

